## Protocol for VR Session

Session Dates:

Updated: | Legend: “xxx” are what the experimenter should say

Note: Always refer to the online version for most updated information. For study background, see Research plan

## Praparation one day before

- Check all compartments of the HTC Vive system in the box and recharge two controllers

- Ensure proper functioning of VR computer tower + external monitor + keyboard + mouse

- Ensure one outlet extender (in total we need 6 power outlets minimum), one mini-display-to-display port cable (VR link box to computer), hygiene products (VR masks, wipes)

- Prepare and recharge two laptops for the questionnaire; save questionnaire links on the desktop

- Print out the most recent protocol, schedule, and a sign “VR Klassenzimmer: Klopfen zum Eintreten”

## Praparation before Participant arrives

Experimenter A will be responsible for VR system control; Experimenter B will be responsible for time tracking, communicating with the participant and handling questionnaires (if any)

### Experimenter A+B: Arrange the Room

- Move aside all furniture to leave enough play space at the center (minimum 3m × 3m)

- Put 3–4 chairs outside in the waiting area

### Experimenter B: Other Preparations

- Prepare hygiene products

- Prepare the printed schedule and protocol

- Put the sign on the room door

### Experimenter A: Tech Preparations

##### Set up the VR equipment

- Set up VR base stations

  - Attach the base stations to the ceiling mount; be careful not to screw it in too tightly

  - Make sure the stations can “see” each other and tilted a little to the floor

  - Connect their power

  - Indicator lights should be green

    - If using Vive and not Vive Pro Eye, one base station should display b, the other c (not a + b)

- Connect the headset to the computer through the link box

  - mini-display to display port cable: MUST plug in to the display port socket at the top part of the back panel of the computer marked as GPU 3080

  - USB cable: can be connected to any sockets

  - Press the blue button on the VR cable box to turn it on; should see a green indicator light

- Connect VR controllers and questionnaire laptops to power chargers

##### Set up the PC and external displays

- Turn on the smart board and the computer

- Connect the computer to the first external display (office monitor; should be the primary monitor) through HDMI

  - HDMI cable: plug in to the HDMI socket located at the SAME GPU 3080 panel connected to the display port cable from VR link box

  - Double-check that the primary monitor used by the computer and the headset is using the same GPU

- Connect the computer to the second external display (smart board) through HDMI

- HDMI cable: plug in to the HDMI socket located at the GPU 4070 panel

- Start the following software in no given order: SteamVR, SRaniparuntime, Unity Hub, Ollama, DWagent, OBS

  - Click yes to any pop-up windows for permissions

- In Unity Hub

  - Start the VR classroom project

- In SteamVR

  - Check if all VR components are connected; the icons should be green

  - Optional: turn on VR view and performance graph

  - Connect the controllers and perform room setup from the SteamVR menu

    - Always connect the controller marked with “R” (right-hand side) first

  - Ensure correct audio output (2- NVIDIA High Definition Audio) and input (Vive Pro Multimedia Audio) sources in the system setting

  - Test microphone recording, make sure input volume is not 0 (not muted)

    - If muted, press the mute/unmute button on the headset

- In OBS

  - Adjust the size of the capture area in the VR session scene

  - Direct the OBS recording output folder to Videos/current date (create one if non-existent)

- Perform a test recording before participant shows up

  - Enter play mode in the VR classroom, after a short while of loading, you’ll see and able to adjust the configuration menu

    - Provider: Ollama (local); Model: gemma:3; Language: German; Context words limit: 20000; Additional rules: copy and paste from the schedule.xlsx

    - When the configuration menu is shown but the start button hasn’t been pressed, the user can see and move around in VR, but the AI conversational agent feature is not activated

  - In OBS, start the recording and check whether the “Desktop Audio” (output) is recording

    - if not, manually change the out source to 2- NVIDIA High Definition Audio instead of the default

  - Speak near the mic on VR headset to check whether the Mic/Aux’s device (input) is recording

    - if not, manually change the input source to Vive Pro Multimedia Audio instead of the default

  - Make sure the people in the room can also hear what’s happening in the VR through smart board

    - if not, check SteamVR→Einstellungen→Audio→Audiospiegelung (audio mirror)→change to SMART; adjust smart board volume if needed

## After Participant arrives

Timeline: 10min simulation for each participant; 5min peer feedback and discussion for each participant; with 10min buffer until next slot

### Preparation

- Experimenter B:

  - Communicate with participants about the timeline

- Experimenter A:

  - Clean the headset used by the last participant; rename video files from last participant; ready the hardware and softwares

### Conduct VR simulation

- Experimenter B:

  - Briefly explains VR (more detailed instruction in the VR through the audio instruction), help the participant put on the headset

    - "This might be your first-time experiencing VR, this environment is totally safe and we are looking out for you in real-life. Please move inside the boundary of the virtual wall indicated by blue grids, within this boundary, you can move around freely without worries. But if you are feeling uncomfortable or overwhelmed at any point, just let us know.”

    - “Once you put on the headset, the VR classroom takes a few seconds to load, then you'll hear detailed audio instructions. The audio instruction ends with "Viel Spaß”, that’s your sign to begin your lesson. You’ll also hear an announcement to end the simulation “Die Unterrichtseinheit ist nun beendet””

    - “Please try to speak slowly and clearly to your students.”

    - “Here’s a hygiene facemask, please wear it like a batman mask. Now I’m going to put this headset around your head, let me know if it’s too tight or too loose.”

  - Perform eye calibration procedure for the participant to ensure visual acuity (optional).

    - “To ensure the best visual quality, I will perform a short procedure to track your eye movements. Please stand still and face the front.”

  - Make sure controllers are connected (blue indicator light) then hand the participant the controllers (make sure the right-hand controller is indeed in their right hand):

    - “Next, please stretch out both of your hands, I’m going to put two controllers in your hand. Please put your thumb on the touch pad, and your index finger on the trigger. Please look down at your hands, you should see whether the virtual hand is matching your actual hand.”

    - “Finally, we’ll put you into the VR classroom and you’ll hear an audio instruction." OR “Please use Left Touchpad to teleport; Right Trigger to talk to the students, like using a walkie-talkie.”

- Experimenter A:

    - While Experimenter B is communicating with the participant

      - Press the play button in Unity; then configure the simulation

    - Once the participant puts on the headset successfully,

      - click "start recording" in OBS, make sure that both the desktop audio and Mic/Aux’s device are recording (remember to start recording before starting the simulation)

      - in Unity, press “Start” in the configuration menu

    - Continue to monitor Unity console, performance graph, and video recording throughout out the session, note down any issues

### Posttest

- Experimenter A:

  - help the participant to take off the headset

  - Stop recording and press stop button in Unity

  - Append participants’ family name to the recording name: e.g., 2024-07-01_10_53_Ali.mp4

  - Check off the participant's name in the participant list, and take a note if anything noteworthy happened

  - Change headset cover and sanitize the controllers

- Experimenter B:

  - Instruct participant for the next step (feedback, and/or questionnaire)

### End of the day

Put the furniture back in place and return the room key.

### Trouble Shooting

- Icons in Steam VR are grey and not green

  - If all icons are grey, check whether the display port cable is connected to the right socket: should be the one in the GPU 3080

  - If headset icon is grey, check whether the indicator light on the VR cable box is green, if not, press the button on the box once

  - If controllers are grey, check whether the controllers are connected (indicator light on controllers are green), if not, long press the bottom button on the controller to turn it on, or reconnect the controller through blue tooth, or recharge the controllers

- I cannot see what’s going on in VR from the external display (smart board)

  - Click the ≡ symbol on the top left of Steam VR, choose “VR view”, you should be able to see what’s showing in VR now

- The mic channel in OBS didn’t pick up participant’s sound

  - Click the ⋮ symbol on the right of Mic, check whether the input source is set to HTC vive

  - If this doesn’t work, check the system audio to see whether the mic has been muted

  - If this doesn’t work, press the mute button on the right speaker of the headset.

- People in the room cannot hear what’s going on in the VR

- SteamVR→Einstellungen→Audio→Audiospiegelung→SMART

- If none of the problems/solutions list here is applicable, restart Unity Hub, SteamVR or the computer