// Code generated by senseBox Blockly on Mon Apr 20 2026 10:42:31 GMT+0200 (Mitteleuropäische Sommerzeit)

#include <senseBoxIO.h>

#include <SPI.h>

#include <Wire.h>

#include <Adafruit_GFX.h> // http://librarymanager/All#Adafruit_GFX_Library

#include <Adafruit_SSD1306.h> // http://librarymanager/All#Adafruit_SSD1306

#include <SD.h>

#include <LTR329.h>

#include <SparkFun_SCD30_Arduino_Library.h> // http://librarymanager/All#SparkFun_SCD30_Arduino_Library

const long intervalInterval = 10000;

long time_startInterval = 0;

long time_actualInterval = 0;

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_RESET -1

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

File sdFile;

bool lightsensortype = 0; //0 for tsl - 1 for ltr

//settings for LTR sensor

LTR329 LTR;

unsigned char gain = 1;

unsigned char integrationTime = 0;

unsigned char measurementRate = 3;

SCD30 airSensor;

int read_reg(byte address, uint8_t reg)

{

int i = 0;

Wire.beginTransmission(address);

Wire.write(reg);

Wire.endTransmission();

Wire.requestFrom((uint8_t)address, (uint8_t)1);

delay(1);

if(Wire.available())

i = Wire.read();

return i;

}

void write_reg(byte address, uint8_t reg, uint8_t val)

{

Wire.beginTransmission(address);

Wire.write(reg);

Wire.write(val);

Wire.endTransmission();

}

void Lightsensor_begin()

{

unsigned int u = 0;

u = read_reg(0x29, 0x80 | 0x0A); //id register

if ((u & 0xF0) == 0xA0)            // TSL45315

{

write_reg(0x29, 0x80 | 0x00, 0x03); //control: power on

write_reg(0x29, 0x80 | 0x01, 0x02); //config: M=4 T=100ms

delay(120);

lightsensortype = 0; //TSL45315

}

else

{

LTR.begin();

LTR.setControl(gain, false, false);

LTR.setMeasurementRate(integrationTime, measurementRate);

LTR.setPowerUp(); //power on with default settings

delay(10); //Wait 10 ms (max) - wakeup time from standby

lightsensortype = 1;                     //

}

}

uint32_t Lightsensor_getIlluminance()

{

unsigned int lux = 0;

if (lightsensortype == 0) // TSL45315

{

unsigned int u = (read_reg(0x29, 0x80 | 0x04) << 0);  //data low

u |= (read_reg(0x29, 0x80 | 0x05) << 8); //data high

lux = u * 4; // calc lux with M=4 and T=100ms

}

else if (lightsensortype == 1) //LTR-329ALS-01

{

delay(100);

unsigned int data0, data1;

for (int i = 0; i < 5; i++) {

if (LTR.getData(data0, data1)) {

if(LTR.getLux(gain, integrationTime, data0, data1, lux));

if(lux > 0) break;

else delay(10);

}

else {

byte error = LTR.getError();

}

}

}

return lux;

}

void setup() {

Wire.begin();

display.begin(SSD1306_SWITCHCAPVCC, 0x3D);

display.display();

delay(100);

display.clearDisplay();

SD.begin(28);

sdFile= SD.open("CO2.txt", FILE_WRITE);

sdFile.close();

sdFile = SD.open("CO2.txt", FILE_WRITE);

sdFile.println((String("Zeit in s") + String(";") + String("Beleuchtungsstaerke in lux") + String(";") + String("Temperatur in °C") + String(";") + String("CO2-Konzentration in ppm")));

sdFile.close();

Lightsensor_begin();

if (airSensor.begin() == false)

{

while (1)

;

}

airSensor.useStaleData(true);

}

void loop() {

time_startInterval = millis();

if (( millis() <= 1800000)) {

if (time_startInterval > time_actualInterval + intervalInterval) {

time_actualInterval = millis();

display.clearDisplay();

display.setCursor(0,10);

display.setTextSize(1);

display.setTextColor(WHITE,BLACK);

display.println((String("Zeit in s") + String(millis() / 1000) + String("Beleuchtungsstaerke in lux:") + String(Lightsensor_getIlluminance()) + String("Temperatur in °C:") + String(airSensor.getTemperature()) + String("CO2-Konzentration in ppm:") + String(airSensor.getCO2())));

display.display();

sdFile = SD.open("CO2.txt", FILE_WRITE);

sdFile.println((String(millis() / 1000) + String(";") + String(Lightsensor_getIlluminance()) + String(";") + String(airSensor.getTemperature()) + String(";") + String(airSensor.getCO2())));

sdFile.close();

}

}

else {

display.clearDisplay();

display.setCursor(0,0);

display.setTextSize(2);

display.setTextColor(WHITE,BLACK);

display.println("Messung abgeschlossen");

display.display();

while(true);

}

}